Example: Water pump efficiency

Calculate the pump’s efficiency for a small irrigation pump, such as a sprinkler pump of 1 HP with the following specs (e.g., Flotec Thermoplastic Sprinkler Pump 1 HP):

Horsepower				1

Gallons Per Minute (GPM)		55

Best for use at maximum depth of 	<20' or 6m

Units & Unit Conversions involved:

1 Ga = 3.79 L

Mass of one liter of water;	ml = 1 kg

Mass of 1 gallon of water;	mg = 3.79 kg

Watts ≡ kg. m2 / s3

P=Wt= ∆E∆t= ∆mgh∆t =g h  ∆m∆t
∆m∆t= pgh= 1.0 hp746 W1 hp9.8 m.s-26m=1.0 hp746 kg.m2 . s-31 hp9.8 m.s-26m

=12.69 kg.s-1 = 12.69  kg.s-11 Gallon3.79 kg

= 3.34 GPS = 200.85 GPM (this is Gallon per minute assuming 100 % efficiency

Calculated Water Pump efficiency =55 GPM200.85 GPM *100=27.38%

SumpMarine UTP 115-Volt 330 GPH Portable Transfer Water Pump, Bronze

Fpower Portable Transfer Water Pump Battery Powered Self-Priming Pump -12V, 300 GPH, 3/4" Garden Hose

Example: Water Pump Power

A water pump system must be able to raise 8 gallons of water every minutes up to a height of 10 m (pumping water from 10 m underground water source to the ground level).  The density of the water is 1.04 g/cm3, due to the minerals in the water.

- What is the work done to lift one liter of the water?

- What is the operating power of the water pump, assuming its efficiency is rated as 23.5 %?

Density		 D=1.04gcm3 1 cm31 ml1000 ml1 l1 kg1000 g=1.04kgl= m1l

m = 1.04 kg

F = mg = (1.04 kg) (-9.8 m/s2) = -10.2 N

Therefore;	W = (10.2 N) (10 m) = 102 J	Work done to raise one liter of water by 10 m

Volume = 8 Gallons = (8 Gallons) 3.79 l1 Gallon=30.32 l

The work done to move 8 gallons or 45.48 l of water is then = (102 J) (30.32) = 3092.6 J

Leading to	P= wt= 3092.6 J60 sec=51.54 Watts=51.54 Watts1 hp745.7 Watts=0.069 hp

Assuming 23.0% efficiency for the water pumping system the electric power needed to run the pump is;                                                 		  0.069 hp0.23= 0.3 hp

Example: Water Pump Operation

Could a 1.0 horsepower (hp) water pump can pump 10 Gallons per minute from a maximum vertical depth of 10m, if the pump efficiency is 50%?

Units & Unit Conversions involved:

1 Ga = 3.79 L

Mass of one liter of water;  ml = 1 kg

Mass of 1 gallon of water;  mg = 3.79 kg

Watts ≡ kg. m2 / s3

P= Wt = ∆E∆t= ∆(mgh)∆t=gh ∆m∆t

∆m∆t= pgh

∆m∆t= 1.0 hp(746 W)1 hp(9.8ms2 )(10 m)= 1.0 hp(746 kg. m2 / s31 hp(9.8ms2 )(10 m)

=7.61 kg /s

= (7.61 kg/s)(1 Gallon / 3.79 kg)

= 2.00 g/s = 120 g/m	assuming a pump with 100% efficiency

= 60 g/m at 		assuming a pump with 50% efficiency

I psi = 6.894757 kPa

https://www.globalspec.com/SpecSearch/SearchForm/flow_control_flow_transfer/pumps/water_pumps